# Work Instruction WI-BRK-100-A  Rev C

Product: BRK-100 Motor Bracket Assembly    Cell: 3    Takt time: 4 min

# Safety

Wear safety glasses. Keep hands clear of the arbor press ram while pressing.

Do not use press punch PT-301 if the pilot is chipped or worn (see NCR-0042).

# Tools required

| Tool ID | Description | Location |
| --- | --- | --- |
| FX-201 | Locating Fixture | Cell 3, bench A |
| PT-301 | Bushing Press Punch | Cell 3, press AP-501 |
| GA-401 | Go/No-Go Pin Gauge | Cell 3, gauge board |
| TW-101 | Torque Wrench 2-25 Nm | Cell 3, tool board |
| AP-501 | Arbor Press 1 t | Cell 3, bench A |

# Assembly steps

| Step | Operation | Tools |
| --- | --- | --- |
| 10 | Clean locating fixture FX-201 and check both dowel pins for burrs or damage. | FX-201 |
| 20 | Place bracket BRK-100 in the FX-201 nest so that both dowel pins enter the bracket holes. The bracket must sit flat on the pocket floor. | FX-201 |
| 30 | Put bushing BU-12 on the pilot of press punch PT-301. Press it into the bracket bore with arbor press AP-501 until the punch shoulder seats on the bushing flange. Maximum force 4 kN. | PT-301, AP-501 |
| 40 | Check the bushing bore with pin gauge GA-401: the GO end (D10.00) must enter freely, the NO-GO end (D10.05) must not enter. Record the result in the inspection log. | GA-401 |
| 50 | Fit motor plate MP-50. Apply medium-strength threadlocker and tighten the 4 x M6 screws SC-616 to 9.5 Nm in a cross pattern with torque wrench TW-101. | TW-101 |
| 60 | Lift the assembly out of FX-201 using the finger slots and attach the serial number label. | FX-201 |

# Revision history

Rev A  2025-11-03  First issue.

Rev B  2026-04-22  Torque changed from 8 Nm to 9.5 Nm (ECR-118).

Rev C  2026-09-21  Weekly pilot check of PT-301 added after NCR-0042.